| Risk Assessment |  |  |
| --- | --- | --- |
| Service: Countryside Service | Assessment Reference: CSRA-DPCP | Date: 01/08/22 |
| Risk Assessment Title: Dinton Pastures Country Park |  |  |
| Additional Information: For risks and controls related to fishing, paddling pool, play area and specific organised events including Marvellous Festival, please see relevant separate risk assessments. |  |  |
| Review Date: 01/08/23 | Risk Owner: Simon Bartlam |  |
| Risk Assessment Team (names and job titles): Finlay Rothery (Risk assessor and Estate Worker – Technical Support) |  |  |

| Ref No. | What are the hazards? How could they cause harm? What are the possible injuries/illnesses? | Who could be harmed? (e.g. colleagues, contractors, visitors, member of public) | What control measures are already in place? | Risk Low Medium High | Detail any additional control measures needed (add to action plan for implementation) | Revised Risk Low Medium High | Who will action: | By When? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Hazard Use of vehicles on site by CS staff and contractors How could they cause harm? Collision with pedestrians, collisions with natural features, getting stuck Injuries/illnesses? Minor and major injuries | Staff, contractors, members of the public | - Walking pace when driving on site. - Hazard lights on. - Stop operation to allow pedestrians to pass. - RAMS for contractor works on site. | Low |  |  |  |  |
| 2 | Hazard Car Park How could they cause harm? Collision with pedestrians, collisions with natural features, cars parked blocking emergency vehicle access Injuries/illnesses? Minor and major injuries | Staff, contractors, members of the public | - 10mph speed limit signs around car park. - Speed bumps on driveway. - Pedestrian crossings and footpaths in convenient areas. - Double yellow lines on driveway to prevent parking. - Marked parking bays to prevent uncontrolled parking positions. | Low |  |  |  |  |
| 3 | Hazard Bicycles How could they cause harm? Collisions between bicycles and pedestrians, animals Injuries/illnesses? Minor and major injuries | Members of the public, staff, contractors | - Code of conduct for cyclists on website. - Dogs on lead/under control signage on greenways. | Low |  |  |  |  |
| 4 | Hazard Overhead power lines How could they cause harm? Machinery making contact with powerlines, trees falling on powerlines, visitors climbing pylons Injuries/illnesses Electrocution, physical injuries | Members of the public, staff, contractors | - No use of large extending machinery in vicinity of powerlines. - Physical restrictions on bases of pylons. - Utilities companies maintain vegetation around and under powerlines. - Contractors and staff to be made aware of locations of powerlines before commencing works. - No kite flying, drones or model planes allowed on site (except by permission of Countryside Officer). | Medium |  |  |  |  |
| 5 | Hazard Public access to watercourse How could they cause harm? Erosion of bridged culverts, flooding, fast-moving water, falling in, swimming Injuries/illnesses Injuries from falling from bridges or banks into watercourse, being swept away, drowning | Members of the public, staff, contractors | - Annual site inspection to check for problems. - Revetment works carried out on eroded banks and culverts when considered dangerous. - No swimming signage on lake front and by Loddon Bridge and Bader Way bridge. - Regular site patrols on hot days. - All water activities run through DAC, with separate risk assessments. - Self-closing gates preventing clear access from play park to lake. - Water sports allowed on black swan only with permission by Dinton Activity Centre. | Medium |  |  |  |  |
| 6 | Hazard Reservoir How could they cause harm? Damage to bank and dam wall from scouring, erosion and flooding Damage to sluice Breach of dam wall leading to flooding downstream Injuries/illnesses? Flooding, damage to property, drowning | Members of the public, staff, contractors | - Weekly inspection by countryside officer/estate team staff. - Daily inspection of circular route by duty ranger. - Scouring identified and repaired immediately. - Annual maintenance to combat erosion. - Twice yearly inspection by civil engineer contractor. - Removal of all trees from around outlet/sheet piling. | Medium |  |  |  |  |
| 7 | Hazard Mains gas and water pipes, underground electric cables How could they cause harm? Water or gas leaks from digging/excavating, damage to electrical cables Injuries/illnesses Injury from high pressure water, damage to infrastructure, explosions, or fire. | Members of the public, staff, contractors | - Inform contractors/staff working on site of locations of mains pipes. - Management plan contains information about locations of mains. - Emergency contact details available if damage caused to mains. - No excavation in vicinity to gas mains – special permissions required as detailed by SGN. - No metal detectors allowed on site. | Low |  |  |  |  |
| 8 | Hazard High pressure sewers How could they cause harm? Sewage leaks from digging/excavating Injuries/illnesses Contamination, disease | Members of the public, staff, contractors | - Sewer lines marked in management plan and provided to contractors working on site. - No excavation on sewer line unless by approved contractors performing repairs/maintenance on sewerage system. - Emergency contact on pumping station. | Low |  |  |  |  |
| 9 | Hazard Flashers/assailants/ attackers How could they cause harm? Perpetrators may hide in vegetation Injuries/illnesses Intimidation, assault injuries, trauma | Members of the public, staff, contractors | - Clear sight lines around the site where possible. | Low |  |  |  |  |
| 10 | Hazard Extreme weather How could they cause harm? Major storms causing falling trees, snow and ice Injuries/illnesses Slipping, crushing injuries. Minor and major injuries | Members of the public, staff, contractors | - No work to be carried out on site in extreme weather. - Access to car park may be closed if car park deemed unsafe to public. - Car parks and driveway snow-ploughed and gritted where possible. - Café access ways and in front of ticket machines gritted in icy conditions. - Ice warning signs up around park in winter months. | Medium |  |  |  |  |
| 11 | Hazard Slips, trips and falls How could they cause harm? Uneven ground, potholes, slippery surfaces after rain Injuries/illnesses Minor injuries, Fractures, Spinal injuries, Head trauma | Members of the public, staff, contractors | - Annual inspection of footpaths. - Issues reported by the public inspected promptly by CS staff. | Medium |  |  |  |  |
| 12 | Hazard Antisocial behaviour How could they cause harm? Vandalism, aggression towards staff/visitors, excessive alcohol or drug consumption, use of motorbikes on site Injuries/illnesses Minor injuries, assault injuries, major injuries, visitor discomfort, fear and distress | Members of the public, staff, contractors | - No alcohol allowed in play park. - Drug users asked to leave site if identified by CS staff. - Antisocial behaviour referred to police if CS staff are unable to stop it safely. - Daily site checks for vandalism, damaged infrastructure. - Reports from the public addressed in a timely manner. | Low |  |  |  |  |
| 13 | Hazard Trees and vegetation How could they cause harm? Falling branches, falling trees, overgrown vegetation blocking paths, giant hogweed growth Injuries/illnesses Cuts, scratches, thorns, hogweed burns, eye injuries, head injuries, crushing injuries, death | Members of the public, staff, contractors | - Annual inspection of trees along footpaths (level 1). - Dead/dangerous trees removed. - Vegetation cut back from foot paths through growing season. - Giant hogweed removed if found near to footpaths. | Medium |  |  |  |  |
| 14 | Hazard Fires, BBQs How could they cause harm? Uncontrolled spread of fire Injuries/illnesses Burns, smoke inhalation, death | Members of the public, staff, contractors | - No fires or BBQs on site (except bonfires by CS staff). - Anglers may have small gas stoves or BBQs, but no open fires. - Fire extinguishers in café and CS buildings – only to be used when fire is small or blocking escape. | Medium |  |  |  |  |
| 15 | Hazard Litter, dog poo, needles How could they cause harm? Contact with harmful waste/litter Injuries/illnesses Needlestick injuries, contamination | Members of the public, staff, contractors | - Weekly site patrol by DR with litter picker. - CS staff aware of safe handling procedures for sharps/litter. | Low |  |  |  |  |
| 16 | Hazard Grazing animals How could they cause harm? Attack by cattle, escaping cattle Injuries/illnesses Minor injuries, major injuries | Members of the public, staff, contractors | - Signage informing visitors of presence of grazing animals, and advice on appropriate behaviour around livestock. - Docile breeds only. - No bulls. - Fence lines inspected by grazier before livestock put into field. | Low |  |  |  |  |
| 17 | Hazard Terrorist attack How could they cause harm? Staff physically assaulted, shot, explosions Injuries/illnesses Minor injuries, major injuries, death | Members of the public, staff, contractors | - Staff trained in procedures in case of terrorism event. - Run Hide Tell - Staff not to place themselves in harm’s way. | Low |  |  |  |  |
| 18 | Hazard Bridges How could they cause harm? Damage or vandalism to bridge, side slats missing Injuries/illnesses Minor injuries, major injuries, drowning | Members of the public, staff, contractors | - All staff to be made aware of weight limit of bridges. - Loddon bridge inspected for damage or vandalism daily by duty ranger. - Highways to carry out two yearly general inspections - Highways to carry out six yearly principal inspections based on WBC policy-BD63/07 Inspection of Highway Structures | Low |  |  |  |  |
|  | Hazard Site furniture (benches, fences, platforms etc.) How could they cause harm? Damaged furniture collapse, vandalism, rotting wood Injuries/illnesses Minor injuries, major injuries | Members of the public, staff, contractors | - Annual inspection of site furniture including benches, fences and bridges. - Daily patrol by duty ranger to check for hazards. | Low |  |  |  |  |

| Ref No. | Action required | Action Owner | Due date | Status / date closed |
| --- | --- | --- | --- | --- |

Risk Owner – I have overall responsibility for the risk assessment. I understand and accept the risks and how they are being managed (mandatory)

| Name: Simon Bartlam | Position: Countryside Officer | Signature: S Bartlam | Date: 10.8.22 |
| --- | --- | --- | --- |

| LIKELIHOOD OF HARM OCCURRING | Certain | Medium | Medium | High | High | High |
| --- | --- | --- | --- | --- | --- | --- |
|  | Very Likely | Low | Medium | High | High | High |
|  | Likely | Low | Medium | Medium | High | High |
|  | Could Happen | Low | Low | Medium | Medium | Medium |
|  | Unlikely | Low | Low | Low | Low | Medium |
|  |  | Minor Injury No first aid or treatment required | Minor Injury First aid on site or treatment off site | Lost Time Injury i.e. time off work | Major Injury | Death or Permanent Disability |
|  |  | POTENTIAL OF HARM OCCURRING |  |  |  |  |

| Summary (delete as appropriate) |  |
| --- | --- |
| Risk without additional controls: | Low / Medium / High |
| Additional control measures required: | Yes / No |
| Risk with additional controls actioned: | Low / Medium / High |

| Priority level | Defining Risk Level | Expected timescale for action |
| --- | --- | --- |
| Low | - Control measures must be maintained and further “reasonably practical” ways of reducing the risk further could be explored - The absence of systems or controls does not present an immediate risk of any injury - A non-compliance with the required standard is identified but which does not represent a legislative breach - Impacts would have little or no impact on the business activities and the reputation of the organisation - An opportunity for enhancement of existing arrangements is identified which is not a non-compliance | - Actions will be medium (3- 6 months) or, - Longer term (6 –12/18 months) Please note - Low priority actions may be assigned short target dates for completion where the action can be completed quickly with minimal cost, especially at a local level |
| Medium | - Action required soonest – interim measures may be needed, and close monitoring is required - Safety critical systems are identified as partially in place but not complete or require enhancement - A failure to adhere to existing required standards e.g., updating risk assessments annually, keeping to safe system of work - The absence of systems or controls does not present an immediate risk of frequent or major injury, multiple numbers of persons injured or fatality. - A minor legislative breach is identified which is unlikely to immediately result in enforcement action. - Circumstances could cause a loss of business - Circumstances which may cause significant property loss or damage | - Actions will be short-term (< 1 month) or, - Medium term (3-6 months) |
| High | - Work should not be started or continued until the risk has been reduced. If it is not possible to reduce the risk even with unlimited resources, then the work has to be prohibited until it is safe to continue, or the risk has been reviewed and accepted by the risk owner - Employees could be fatality injured or sustain life changing injuries in the course of work - A significant breach of legislative duty resulting in public reprimand and financial or criminal penalties - An absence of the required policy or procedure in relation to a safety critical activity - Safety critical controls are absent - Any circumstance observed that presents a risk of imminent or significant harm to personnel, property or service continuity - Circumstances would cause major reputational and financial damage that will result in the Council failing to meet its statutory duties | - Actions will be immediate or short term (< 1 month) Please note - In some cases it will not be practicable to implement actions within a short time scale and interim alternative or emergency arrangements or cessation of an activity should be considered. |