| Date: 9 February 2023 Application: 230074 |  |
| --- | --- |
| WBC Highways | Development Management & Compliance |
|  | P.O. Box 157 |
|  | Shute End, Wokingham |
|  | Berkshire, RG40 1BN |
|  | Tel: (0118) 974 6000 |
|  | Minicom No: (0118) 974 6991 |

Dear WBC Highways,

Outline Planning Approval Consultation

Application Number: 230074

Applicant: Ms Penelope Clayden and

Site Address: Land adjacent to School Road and Orchard Road, Hurst, Reading

Parish:

Grid Reference: Easting - 479664, Northing - 173365

Type of Development: Major Dwellings (10+)

Proposal: Outline planning application for the proposed erection of 24 dwellings with assocaitaed access and 15 carparking spaces for the local Primary school. Access only to be considered (with Appearance, Landscaping, Layout and Scale to be reserved).

Case Officer: Mark Croucher

Development Management has received the above application and we require your comments on the proposal using the recommended memorandum below. The documents associated with this are available to view in Information at Work using the application number 230074. Alternatively, public documents are available to view on the Council’s planning application search page: Wokingham Borough Council Online Planning.

Please index your response into Information at Work against the application. If you are recommending conditions, you should give a reason with reference to relevant policies. A list of standard conditions can be found on the Z Drive at: Z:\Standard Planning Conditions.

Your observations are required in respect of this application within 10 working days of the above date or 15 working days if the development type is classed as a major development.

Yours sincerely,

Development Management & Compliance

M E M O R A N D U M

| From: | Gordon Adam Development Control |  |
| --- | --- | --- |
| Service | WBC Highways |  | App No: | 230074 |
| Address: | Land adjacent to School Road and Orchard Road, Hurst, Reading. |  |  |  |
| Proposal: | Outline planning application for the proposed erection of 24 dwellings with assocaitaed access and 15 carparking spaces for the local Primary school. Access only to be considered(with Appearance, Landscaping, Layout and Scale to be reserved). |  |  |  |
| Type of Development: |  | Major Dwellings (10+) |  |  |
| Site Visit Made: | Yes/No |  |  |  |

| Summary Of Recommendations |  |  |  |
| --- | --- | --- | --- |
| ☐ No comment ☐ No objection ☐ No objection subject to conditions (and reasons) stated below X Request further information before determination as stated below ☐ Objection due to the reason(s) stated below |  |  |  |
| Comments On Proposal |  |  |  |
| General - Highways is disappointed that no pre-application discussions took place on this application. - This application is supported by a transport statement. - Highways is of the opinion that this is an amended application due to the change from two residential accesses to one. - Highways comments on the initial application are still outstanding. Parking - A parking calculator has been submitted together with an illustrative masterplan and housing mix. This is welcome. - Please note that Hurst is not classed as urban but as a village in the calculator. Calculator would need to be resubmitted accordingly. - In addition, plot 13 appears to have one parking space but has been included as having 2 spaces in the calculator. This will need to be clarified. - In addition, the transport statement states that the affordable units would have unallocated parking, however the calculator shows 1 allocated space each. This needs clarifying. - In addition, it is proposed that there would be 15 spaces provided for the school to provide an area for drop off and pick up. This is welcome as it would alleviate parking problems on School Road. - However, there is no explanation on how drivers would be encouraged to use this parking area instead of parking on School Road. Parents appear not to use the Village Hall car park for drop off and pick up. - The parking for the school will be managed by a Management Company, however there is no information on whether it would be gated to stop inconsiderate parking or whether there would be a height barrier. - The additional spaces provided for the school could also serve visitors to the Village Hall. - In addition to this the spaces that have been provided for the houses and flats have not been labelled, so it is not clear who these spaces are assigned to. - It appears that all spaces are 5m x 2.5m and there is ample space behind for safe manoeuvring. - All garages would need to have internal dimensions of 6m x 3m or 7m x 3m if cycle storage is to be included. - All parking spaces in front of garages would need to be set back 0.5m. Whilst most plots have sufficient space, the driveway for plot 4 is too short. Cycle Parking - It is proposed that cycle parking will be to borough standards which is welcome. - The cycle storage will either be in garages (internal dimensions 7m x 3m) or in rear gardens. These would need to have direct access. - For the flats, it is proposed that there would be a communal cycle storage. This is welcome and each space would need to be 2m x 0.9m. This building would need to be secure, sheltered and accessible. - Highways would also recommend a Sheffield stand to be provided close to the main access to the block for visitors. - Highways is satisfied with the proposed level of cycle parking and that cycling can be included as a reserve matter. EV Charging - Electric vehicle charging will be provided in accordance with Building Regulations and shall consist of a minimum of 1 x active EVC point per unit which is welcome. - Details of the provision would be secured through a planning condition. Trip rates - Highways would have expected that the approved borough trip rates should have been used to assess this application. - Highways have carried out a comparative assessment and the difference between the trip rates is very small. - There the level of trips from this site are acceptable and there would be little impact on the wider highway network. Access - It is proposed that there will be 2 accesses on School Road, one for the school car park and the other for all 24 dwellings. - The access to the school car park would be via a 6m crossover. There is no mention of gates or whether or not there would be a height barrier. - The access to the proposed dwellings would be a 5m wide road with a 2m pavement on the east side of the road, - The proposed access would result in the loss of 3 trees which are part of a group of Trees TPO. - The parking surveys around entry and exit times indicate that parking could have an impact on the proposed access for the dwellings and the car park and measures would be required to ensure that there is safe operation of this access through eg parking controls which would also persuade the car park to be used. - The proposed access would be around 20m from the junction of Tape Lane and School Road. Due to the number of vehicles using Tape Lane, the layout of this junction and the low level of traffic generated by the site there would be no highway impact. No mention of any safety concern was included in the Road Safety Audit. Visibility Splays - Automatic traffic counts were installed on School Road and Orchard Road to collected speed data to inform visibility splays which is welcome. - The lengths of the visibility splays appear to be the right order for the 85th percentile speeds. The visibility splay calculations from MfS have been submitted and these are acceptable. - The visibility splays are acceptable for both the access to the dwellings and the school car park. - Any landscaping with the splays would need to be maintained to be less than 0.6m. This will be secured through a planning condition. Swept path analysis - Swept path assessments for a refuse vehicle, fire tender and box van with acceptable dimensions have been submitted. - For the dwellings access road, all turning movements would need to be submitted not just left in and left out. - Highways would need to see a refuse vehicle passing a large car on the site road. - Highways would have expected to see swept paths of a large car accessing/egressing the end spaces in the proposed school car park. Road Safety Audit - A Road Safety Audit Stage 1 and Designer’s comments have been submitted which is welcome. - There was only one issue raised in the audit which has been accepted by the designer and will be included in detailed design which is acceptable. - However, due to the location of the site close to the school, no site visit was undertaken during school entry and exit times. Site Roads - There has been no mention on whether or not the site roads would be offered for adoption or not. It has been proposed that the sites roads would be built to adoptable standards which is welcome. - If the site roads are to be adopted, these would need to be delivered through s38 agreement with the Borough. The Borough would also require agreement on inspection fees and commuted sums. The adoption plans, fees and commuted sums would need to be included in the s106 agreement. - If the site roads are to remain private, this would also need to be included in the s106 agreement, including details of the management company set up to look after the maintenance of the roads, the Borough’s inspection fee and the APC Bond. - If planning approval is granted the s106 would include both private and adopted as an either/or situation with a decision reached prior to submission of a reserved matter application. - It is not clear from the submitted plan ITB18301-GA-001 where the service margins are. However, if it is proposed that the service margin widths would follow WBC - Roads with dwellings fronting 2m - Roads with no dwellings but lighting 1m - Roads with no dwellings and no lighting 0.5m - No trees or bushes would be allowed in service margins, Highways is content for service margins to be included in any reserve matters application. Walking and Cycling - A walking and cycling audit has been submitted which is welcome and recommends minor improvements to the pedestrian routes through the village which is welcome. - These will be secured through a walking and cycling strategy planning condition. - It is proposed that there would be a 2m wide pedestrian route between the through the site which is welcome. Sustainability - A sustainability assessment has been undertaken which is welcome. - Highways accepts that there are limited local services with an acceptable walk distance, however there is no employment opportunities in the village and future residents would need to commute out of the village for work as well as for supermarkets, doctor’s and senior schools. - There is a bus service that runs along School Road that offers the opportunity for bus travel to Wokingham and Twyford and possible connections with train services. However, this service is limited and does not offer any evening services or Sunday services. Saturday services are every 2 hours and would not be available for anyone working on Saturdays. - It is unlikely that there would be alternative modes to the car and that the car would remain the main mode. - Please note that at a recent appeal at Sawpit Road site (four houses), which is slightly further west of this site, concluded that that site was sustainable. - However, at a more recent planning appeal at Land East of Lodge Road, further up Tape Lane, the inspector concluded that that site was in an unsustainable location based on the lack of alternatives to the motor car and that no improvements or contributions could improve the sustainability. Framework Construction Management Plan - Highways would have expected a framework construction management plan to have been submitted. No document was submitted, and this is required. - A framework Construction Management plan has been submitted which is welcome. It is recognised that no contractor has been appointed and therefore the principles included in the framework document would need to be included in a full construction management plan. - All deliveries will be managed and be outside the peak hours which is welcome. - Banksmen will be used to manage construction traffic accessing/egressing the site which is welcome. - The proposed hours of construction are also welcome and will be secured by planning condition. - It is proposed that there will be wheel washing to ensure debris is not carried onto the road, this is welcome. In addition, highways would also need to see included a monitoring of the highway network in the vicinity of the construction access and the use of mechanical street sweepers if required. - It is well that all parking of contractors would need to be included on the site. - Any hoarding would need to be off highway and the gate set back the length of the largest vehicle to access the site. - Highways would have expected to see an indicative construction route map to the site. - There would also need to be an assessment of the condition of the highway prior to and after construction and any re-instatement would need to be agreed between developer and WBC. This will need to be included in any s106 agreement. - If the above comments are included, highways would be content to secure the full document through a planning condition. |  |  |  |
| Conditions & Reasons (if required) |  |  |  |
| Date: | 6th July 2023 | Signed: | Gordon Adam |