| Date: 9 February 2023 Application: 230074 |  |
| --- | --- |
| WBC Drainage | Development Management & Compliance |
|  | P.O. Box 157 |
|  | Shute End, Wokingham |
|  | Berkshire, RG40 1BN |
|  | Tel: (0118) 974 6000 |
|  | Minicom No: (0118) 974 6991 |

Dear WBC Drainage,

Outline Planning Approval Consultation

Application Number: 230074

Applicant: Ms Penelope Clayden and

Site Address: Land adjacent to School Road and Orchard Road, Hurst, Reading

Parish:

Grid Reference: Easting - 479664, Northing - 173365

Type of Development: Major Dwellings (10+)

Proposal: Outline planning application for the proposed erection of 24 dwellings with associated access and 15 carparking spaces for the local Primary school. Access only to be considered (with Appearance, Landscaping, Layout and Scale to be reserved).

Case Officer: Mark Croucher

Development Management has received the above application and we require your comments on the proposal using the recommended memorandum below. The documents associated with this are available to view in Information at Work using the application number 230074. Alternatively, public documents are available to view on the Council’s planning application search page: Wokingham Borough Council Online Planning.

Please index your response into Information at Work against the application. If you are recommending conditions, you should give a reason with reference to relevant policies. A list of standard conditions can be found on the Z Drive at: Z:\Standard Planning Conditions.

Your observations are required in respect of this application within 10 working days of the above date or 15 working days if the development type is classed as a major development.

Yours sincerely,

Development Management & Compliance

M E M O R A N D U M

| From: | NT |  |
| --- | --- | --- |
| Service | WBC Drainage |  | App No: | 230074 |
| Address: | Land adjacent to School Road and Orchard Road, Hurst, Reading. |  |  |  |
| Proposal: | Outline planning application for the proposed erection of 24 dwellings with assocaitaed access and 15 carparking spaces for the local Primary school. Access only to be considered(with Appearance, Landscaping, Layout and Scale to be reserved). |  |  |  |
| Type of Development: |  | Major Dwellings (10+) |  |  |
| Site Visit Made: | No |  |  |  |

| Summary Of Recommendations |  |  |  |
| --- | --- | --- | --- |
| ☐ No comment ☐ No objection ☒ No objection subject to conditions (and reasons) stated below ☐ Request further information before determination as stated below ☐ Objection due to the reason(s) stated below |  |  |  |
| Comments On Proposal |  |  |  |
| LLFA received this application on 9th February 2023. CampbellReith has produced this Flood Risk Assessment (FRA) and Drainage Strategy to support an outline planning application for the proposed residential development on Orchard Road, Hurst. The development is for the construction of 24 dwellings, including affordable housing, with associated public open space and landscaping together with means of access from the A343. The greenfield site is in Flood Zone 1 and generally at Medium, Low or Very Low risk of flooding from any recorded sources. Paragraph 10 states that available information suggests that the sites bedrock deposits are Lambeth Group (Clay, Silt and Sand) in the northern area of the site and London Clay Formation (Clay, Silt and Sand) underlain by Lambeth Group in the southern area of the site. No site-specific ground investigation works have been undertaken on the site. The proposed SuDS strategy involves the use of permeable paving to manage surface water runoff generated from the site. Infiltration should be used where possible as the main source of discharge from the site. Should on-site infiltration testing prove that soakaway drainage is not viable, a connection to the existing highway drainage system or existing foul water network should be pursued. The proposed surface water drainage system can effectively control all runoff generated within the site and maintain pre-development Greenfield runoff, without increasing flood risk elsewhere. LLFA recommends seeing final surface water and foul water drainage strategy with final layout and recommends no objection to the application subject to conditions written below. Condition (Infiltration) If infiltration /Soakway would be considered as preferred means of discharging surface water then it is necessary to conduct BRE 365 test to check infiltration rates. No development shall take place until a soakaway test's has been carried out in accordance with BRE Digest 365, or such other guidance as may be agreed in writing by the Local Planning Authority (LPA). The results of the tests shall be submitted to and agreed in writing by the LPA. The ground through which infiltration is designed to occur should be unsaturated to a depth of at least 1m below the base of infiltration unit. The scheme shall subsequently be completed in accordance with the approved details before the development is first brought into use/occupied. Condition (Exceedance Flow) Development shall not take place until an exceedance flow routing plan for flows above the 1 in 100+40% climate change event has been submitted to and approved in writing by the Local Planning Authority. The proposed scheme shall identify exceedance flow routes through the development based on proposed topography with flows being directed to highways and areas of public open space. Flow routes through gardens and other areas in private ownership will not be permitted. The scheme shall subsequently be completed in accordance with the approved details before the development is first brought into use/occupied. Condition (Overland flow) The layout of the development site and the drainage system should be designed so that natural low lying areas and overland conveyance pathways are used to manage surface runoff, where appropriate, where they do not pose an unacceptable risk to the new developments or downstream areas/ elsewhere. Where run-off from off-site sources is drained together with the site run-off, the contributing catchment should be modelled as part of drainage system in order to take full account of additional flows. Condition (Attenuation) Prior to the commencement of development details of surface water attenuation/storage works shall be submitted to and approved in writing by the Local Planning Authority. The volume balance requirements should be reviewed to reflect actual development proposal, agreed discharge rate and the extent of impermeable areas and runoff to be generated. The storage can be designed on-line or off-line, either on or below the surface, and should be located within the site. Where practicable, above ground system should be used to enhance biodiversity, increase capacity and for ease and safety of maintenance. The scheme shall subsequently be completed in accordance with the approved details before the development is first brought into use/occupied. |  |  |  |
| Conditions & Reasons (if required) |  |  |  |
| Reason To ensure that the site can be adequately drained. It is important that these details are agreed prior to the commencement of development as any works on site could have implications for drainage in the locality. |  |  |  |
| Date: | 20/03/2023 | Signed: | NT |