We would like to propose the above highlighted route is trialled in order to understand whether a number of the existing difficulties experienced are overcome by using the above route.

It is acknowledged the route is not the most direct, however utilising the crossing on the approach to the crossroads would enable users to cross at a perpendicular location, away from the main crossroads. This allows greater visibility in both directions and is located away from the busier central area, and also provides an alternative which is more in line with what the Guide Dog representative who reviewed the design had requested.

This route would enable to the use of the formal crossing facility outside of the ratepayers hall to be utilised. Once across, visually impaired users then have a raised kerb throughout the route (highlighting the location of the carriageway) and has the added benefit of grass verges/private driveways, which will allow users a change of material underfoot to ensure they can be guided to the relevant tactile paving to assist with crossing the section of Nine Mile Ride.

There are new tactile paving slabs assisting the crossing of Avery Close, which will then allow users to access the Shared Space area of the shop frontage car park.

In this area, we understand the guide dog will ensure the user keeps to the building line and vehicles negotiate this section of parking at very low speeds ensuring a safe environment.

It is accepted however, that additional requirements need to be installed to aid users across the Avery Car park access road and we welcome assistance from Steve in order to improve the layout at this location.

The proposed route is summarised below:

- Exit the alleyway that runs along the Ratepayers Hall property boundary

- Cross Finchampstead Road using the existing formal crossing infrastructure

- Proceed along the Finchampstead Road footpath, around the corner and along Nine Mile Ride (easterly direction)

- Utilise the raised table crossing situated close to No.203 Nine Mile Ride

- Once across proceed west – across Avery Close junction with new tactile installed

- Enter shop frontage shared space area following the building line

- Cross Avery Car park access road (Additional works required here, and request to work with Steve to agree measures and layout).