California Crossroads – Guide Dogs for the Blind site walkthrough feedback

Introduction

Site walkthrough took place on 7th February 2025.

Myself and Dean were in attendance on behalf of Wokingham Borough Council and met with Steve Bowers and a member of The Guide Dogs for the Blind, as well as Steves wife.

Steve’s main concerns related in his journey from his property in to the doctors surgery along Finchampstead Road, and the nearby pharmacy in the parade of shops next to Co-Op.

Steve approaches the crossroads from the alleyway at the side of Ratepayers Hall.

The report refers to Steve’s difficulties throughout, however we can also assume the same difficulties would be experienced by all visually impaired users.

The subsequent locations/summary of issues have been recorded below:

- Access/Exit to Bobs fish and chips private car park

Previous layout included a change in level for the dropped kerb, which illustrated the private car park entrance/exit. As the new layout has been regraded, the level change has been removed and therefore Steve is unaware when he is approaching the access.

The request was made that a small section of corduroy paving be laid, so that Steve can feel underfoot the change in surface, therefore highlighting the location of the car park entrance/exit.

- Bollards along private car park perimeter

The previous layout included a boundary fence line for the private car park, along with pedestrian guardrail on the approach to the central crossing location of the double roundabouts – this provided Steve and his trained assistance dog guidance as to where to stand, and acted as a channel guide to the crossing points.

The new layout has removed all pedestrian guardrail, and the car park permitter fencing has been removed and replaced with a line of bollards.

Unfortunately this layout has removed the physical guides which Steve uses, and therefore when using his stick he is unable to locate any physical measures which informs him as to his location. Additionally, the assistance dog does not view the bollards as a fence line/perimeter and therefore regularly guides Steve between the bollards and into the private car park (where vehicles are continually pulling into/out of the parking spaces.

Request was made to try and provide a more easily recognisable physical measure in which Steve can use his stick to ensure he is not led into the car park between the bollards and stays within the public highway/footpath extents.

- Entrance to Co-Op car parking shared space

The reversal of the Shared Space entry and exit has also led to some issues for Steve as he is unable to know when he is approaching the car park entrance, due to the levels being flush with the footway.

Steve did not previously use this route, and preferred to walk along the raised footpath included within the car park that fronts the Co-Op and Pharmacy, however currently is also experiencing some difficulties at this location which is detailed below.

If this route shown above is to be the preferred option, then again, a request for corduroy paving to highlight the entry lane of the car park has been requested.

- Shop frontage edge along shared space

The previous layout included a segregated raised walkway, which Steve could locate by using his stick. The assistance dog is trained to always be positioned on the left hand side of the person, and would ensure they proceed as close to the building edge as possible.

As the new layout has adopted a shared space philosophy, and the area has been made the same level, Steve finds it difficult to ensure he is travelling away from potential vehicle conflicts, and close to the shops. Additionally, the lack of a raised area means Steve is unable to use his stick properly as a guide, and therefore finds that the dog will often lead Steve away from the shop frontages.

The request here is to install a line of corduroy/or some sort of physical raised measure to guide Steve through the shared space area.

- Crossing of Avery Car Park access road

The previous junction layout included a kerb face, and a section of cobbled stones which provided Steve the ability to know he was approaching the access road, and needed to cross.

Once across the road, there was also a length of cobbled stone which ensured Steve knew he had crossed the carriageway and was on either the footpath or the restaurant access and could turn 90 degress to rejoin the footway and proceed towards the doctors surgery.

The new layout has removed the kerbing, and the cobbled blocks meaning Steve is now unsure when he is approaching the access road.

Vehicles are also now cutting the corner on the way in, and travelling over the blocked pave footway in the process – further discouraging Steve from using this footpath to cross the access road – there are no tactiles here either.

The request here was for measures to be implemented either by way or corduroy paving/physical raised measure to provide Steve with guidance that he has negotiated the shop front and is now about to cross the access road.

Steves preferred route would be to utilise the shared space area as he did before – this allows him access to the doctors surgery and therefore the requests above (particularly Nos 1, 2, 4 & 5 are of greater importance).

Additional concerns discussed:

- Finding the tactiles on the new main crossing approach.

Steve currently struggles to find the tactiles on the main central crossing from outside of Bobs Fish & Chips, and therefore is unsure when to stop and turn to safely negotiate the crossing.

Working with Guide Dogs for the Blind, Steve now attempts to travel to the furthest away point of the tactiles, then turn 90 degress to cross the carriageway. By doing this, Steve is able to cross and even if he deviates slightly when crossing can still then find the tactiles on the adjacent side of the road.

Previously the pedestrian guard rails provided Steve with the physical measures to complete the crossing.

- Reason for not using signalised crossing outside of Ratepayers Hall.

I did ask why Steve doesn’t cross Finchampstead Road using the signalised crossing outside of the Ratepayers Hall.

Guide Dogs for the blind always encourage users to cross at a location in which they would be most visible to road users, and therefore the central crossing provides the greatest visibility.

If Steve was to cross using the signalised crossing, he would then need to cross Nine Mile Ride at a location in which he is not as visible.

Additionally, Steve would then need to listen out for vehicles travelling from 2 approaches (one from the east, and the other from the west). Whereas using the central crossing limits the traffic flow directions to contend with.

- Alignment of tactile queries.

There were a couple of locations where the alignment of the tactile paving was questioned, however this had only been observed by eye. We assured that the tactile paving had been set out using a lazer, and installed in accordance with the design, and string lined based on the lazers finding and therefore we believe the tactiles are aligned properly.

- Request for Zebra crossing

Request for a formal central zebra crossing was raised, and we explained the restrictions that prevented such a crossing being installed.